# Physics – Electricity and Magnetism Unit Test

Grade: 11   |   Duration: 50 minutes   |   Total Marks: 40

Instructions: Answer ALL questions. For multiple choice, circle the letter of the best answer.

# Section A – Multiple Choice (2 marks each)

1. According to Ohm's Law, the current through a conductor is:

A. Directly proportional to the resistance

B. Directly proportional to the voltage across it

C. Inversely proportional to the voltage

D. Independent of voltage

Answer: B

2. A resistor of 10 Ω is connected to a 5 V battery. What is the current? (2 marks)

A. 50 A

B. 2 A

C. 0.5 A

D. 0.2 A

Answer: C

3. Which of the following is the SI unit of electric resistance?

A. Ampere

B. Volt

C. Watt

D. Ohm

Answer: D

4. In a series circuit with three resistors R1 = 2 Ω, R2 = 3 Ω, R3 = 5 Ω, the total resistance is:

A. 1 Ω

B. 10 Ω

C. 15 Ω

D. 30 Ω

Answer: B

5. Two resistors of 6 Ω each are connected in parallel. The equivalent resistance is:

A. 12 Ω

B. 6 Ω

C. 3 Ω

D. 1 Ω

Answer: C

6. Kirchhoff's Current Law states that at any junction in a circuit:

A. The sum of voltages is zero

B. The sum of currents entering equals the sum leaving

C. Current is constant throughout

D. Voltage is constant throughout

Answer: B

7. The electric power dissipated by a resistor R carrying current I is:

A. P = IR

B. P = I²R

C. P = I/R

D. P = R/I²

Answer: B

8. A 60 W bulb is connected to a 120 V supply. What is the resistance of the bulb? (2 marks)

A. 2 Ω

B. 60 Ω

C. 120 Ω

D. 240 Ω

Answer: D

# Section B – Short Answer (8 marks each)

9. State Kirchhoff's Voltage Law and use it to find the unknown voltage in a series circuit with a 12 V battery and two resistors with voltage drops of 4 V and 5 V respectively. (8 marks)

10. A student connects three resistors of 4 Ω, 6 Ω and 12 Ω in parallel across a 12 V battery. Calculate the equivalent resistance, the total current from the battery, and the current through each resistor. Show all working. (8 marks)